ZVALS – Group 10

Autonomous Landing System: STM32 ↔ SpeedyBee Interface Plan

Phase 2 Meeting Notes  |  February 2026

# Overview

The control team will use INAV firmware on the SpeedyBee F405 Mini flight controller. INAV's native MAVLink support allows the STM32F4 companion processor to send autonomous flight commands over a dedicated UART, while the RC transmitter remains fully connected and always retains manual override priority. No firmware modification is required.

# System Architecture

## Normal RC Flight (Unchanged)

- RC Transmitter → RF → RC Receiver → SBUS/CRSF → SpeedyBee UART (Serial RX)

- This signal path is completely stock — identical to any standard FPV drone build

- STM32 is fully idle during manual flight, not involved in this path at all

## Autoland Mode (STM32 Active)

- HackRF SDR → STM32F4 (FreeRTOS) — receives 3 beacon distance values

- STM32 runs triangulation control algorithm to compute Roll/Pitch/Throttle/Yaw commands

- STM32 → MAVLink over UART → SpeedyBee (separate UART from RC receiver)

- INAV accepts MAVLink RC_CHANNELS_OVERRIDE on flight axes; RC transmitter retains AUX/mode channels

## Toggle Mechanism

- Pilot assigns a physical switch on RC transmitter to an AUX channel (e.g. AUX1)

- Switch OFF → INAV uses RC input only, STM32 sends nothing

- Switch ON → STM32 detects mode change via MAVLink telemetry feedback, begins sending autoland commands

- Flipping switch back instantly restores full manual control — no software delay

# Hardware Connection

The STM32F4 connects to the SpeedyBee via 3 soldered wires on a free UART (UART2 or UART3):

| STM32F4 Pin | SpeedyBee Pad | Notes |
| --- | --- | --- |
| TX (UART3) | RX2 or RX3 | STM32 transmits → FC receives |
| RX (UART3) | TX2 or TX3 | FC transmits telemetry → STM32 receives |
| GND | GND | Common ground required |

- No USB connection needed between STM32 and SpeedyBee

- 3.3V TTL logic — both chips are compatible, no level shifting required

- Baud rate: 115200

# INAV Configuration (One-Time Setup)

- Ports tab → Set chosen UART telemetry protocol to MAVLink, baud 115200

- RC receiver remains on its own UART as normal Serial RX — completely unchanged

- Modes tab → Assign AUX channel to autoland trigger mode

- No CLI modifications, no firmware recompilation, no hex file editing required

# STM32 FreeRTOS Task Structure

- Task 1: MAVLink RX — reads telemetry from INAV (altitude, attitude, current flight mode)

- Task 2: RF Distance RX — receives 3 beacon distances from HackRF

- Task 3: Triangulation / Control Algorithm — computes autoland commands

- Task 4: MAVLink TX at 50Hz — only active when autoland mode detected; sends RC_CHANNELS_OVERRIDE

Key detail: sending UINT16_MAX on any channel in RC_CHANNELS_OVERRIDE tells INAV to ignore that channel and defer to the RC transmitter. Only Roll, Pitch, Throttle, and Yaw are overridden during autoland.

# MAVLink Library for STM32

- Use official MAVLink C header-only library — drop headers directly into STM32CubeIDE/PlatformIO project

- No separate compilation step, no external dependencies, fully compatible with FreeRTOS + HAL

- STM32 sends: MAVLink RC_CHANNELS_OVERRIDE (command code #70)

- STM32 receives: HEARTBEAT, SYS_STATUS, ATTITUDE, ALTITUDE for control loop feedback

# Why INAV Over Betaflight For This Use Case

- MAVLink RC_CHANNELS_OVERRIDE is natively supported — no workaround needed

- RC receiver and MAVLink companion link coexist on separate UARTs with no conflict

- Manual override always takes hardware priority — satisfies Phase 2 spec requirement

- Bidirectional MAVLink: STM32 receives live telemetry back on the same UART for closed-loop control

- Compatible with QGroundControl for monitoring during testing

# Reference Links

- INAV MAVLink Telemetry Source: iNavFlight/inav – mavlink.c

- INAV MAVLink Telemetry Docs: iNavFlight/inav – Telemetry.md

- INAV RC via MAVLink (feature tracking): iNavFlight/inav – Issue #6571

- INAV Latest Releases (MAVLink RC support confirmed): iNavFlight/inav – Releases

- ELRS MAVLink Setup Guide: expresslrs.org/software/mavlink

- SpeedyBee F405 Mini Stack – Product Page: speedybee.com

- Controlling INAV drone via MAVLink + companion computer: iNavFlight/inav – Issue #10387